ERP Analysis

- Run 1_Import_Files.m to get .set files

- Run 2_EpochFiles.m

- This epochs the files around the offset (-6 seconds pre-offset to +1 second postoffset), and does some simple preprocessing (rereference to common average, filter 0.1 to 30 Hz plus 48-52 Hz notch filter, downsample to 256 Hz). ICA is not necessary here unless you want to run it again. The data is saved as one concatenated file for all conditions, for each participant.

  - Change the directories for EEGLAB and the data directory to match those on the computer being used

  - If there are any issues with the script, run EEGLAB first

- Data cleaning

- Load each data file individually.

- Eye blinks and eye movements

  - ICA: Reject independent components representing eye blinks and horizontal eye movements (should be the same components as in the ERD analysis).

    - This should be done manually but can be checked using IClabel (settings >90% eye movement, as standard) – should generate the same results and can be scripted

- Noisy electrodes:

  - Scroll through data for any noisy channels which should be interpolated – can use the channel spectra for confirmation of outliers.

    - Save the data file here with a different name, incase the artefact rejection needs to be repeated.

- Noisy trials

  - Manual rejection

    - Scroll through and highlight artifactual periods/trials

  - Semi-automated approach

    - Reject trials surpassing +/-125mv amplitude between -1 to +1 seconds (this length is based on analysis period – modify if different epochs are used)

    - Reject trials with joint probability of over 7 / 7 standard deviations

      - Save the data file as _cleaned.

      - These two bits can be scripted, particularly if the analysis needs to be repeated.

- Note: this can be carried out before or after epoching.

    - Cleaning after epoching:

      - Perks: faster when manually going through, whole trials rejected based on only artefacts within the epochs.

      - Cons: Will need to be repeated if you re-epoch.

- Run 3_ERPFigures.m

- This does the main epoching and analysis steps.

- The cleaned data is loaded and analysis (sub)epochs are computed using EEGLAB (-200 to 800ms pre-stimulus-offset). Baseline correction is carried out (-200 to 0s pre-stimulus-offset). The data for each epoch is saved so it does not need to be recomputed each time the script is run (after running the first time, change to ‘computeEpochs = 0’).

- Data is transformed from sensor data to current source density using FieldTrip. All data is saved as a big file so it does not need to be recomputed each time the script is run (after running the first time, change to ‘computeCSD = 0’).

- Figures are generated – grand average topoplots for each condition, grand average ERPs for each condition, and topoplots in selected time windows.

- If statsExports = 1, data is exported for statistical analysis. The data file will consist of 4 conditions x participant#, representing the mean amplitude during the plottingTimesSelected for the selectedElectrode/s.

  - Change paths for Fieldtrip, chanlocs file, currentDirectory

  - Ensure ‘wname’ matches the name from cleaned data in previous step

  - The baseline period can be amended, but should match the artefact rejection period from previous step

  - After computing CSD, change the ‘mostRecentFile’ to the name of the exported file – AllERP_csd_[date of export].mat

  - Electrodes for plotting and exports can be specified in selectedElectrode. If multiple electrodes are specified, these will be averaged.

- Run 4_BigExport.m

- The previous step generates one export per electrode/cluster and time point. To create one large file, run this, specifying the folderPath where the exports are located. The script will find all export files and combine them into one large table with subheadings of electrode number and time window based on the export file name.

- Proceed to statistical analysis